|  | G:BOX mini 6 | G:BOX mini 9 | iBright |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Applications |  |  |  |  |  |  |
| Gel Documentation | YES | YES | YES |  |  |  |
| Fluorescence | YES | YES | YES |  |  |  |
| Colorimetry/ densitometry | YES | YES | YES |  |  |  |
| Chemi- luminescence | YES | YES | YES |  |  |  |
| IR | YES | YES | YES |  |  |  |
| Stain-free | YES | YES | NO |  |  |  |
| Camera |  |  |  |  |  |  |
| Image resolution | CCD, 6MP, 2750x2200 18MP effective 73% QE at 425nm Peak QE 77% | CCD, 9MP, 3380x2704 27MP effective 73% QE at 425nm Peak QE 77% |  | CCD 9.1MP |  |  |
| Bit depth (gray levels) | 16 bit, 65,536 |  |  | 16 bit, 65,535 |  |  |
| Dynamic range | 4.8 OD |  |  | 4.8 OD |  |  |
| Max exposure time | 23hrs 59mins |  |  | 1 Hour |  |  |
| Lens |  |  |  |  |  |  |
| Type | Motor driven, f/0.95 |  |  | Motor driven f0.95 (25mm) |  |  |
| Max sample size (cm) | 15x12 cm |  |  |  | 22.5x18cm |  |
| Min sample size (cm) | 10x8cm |  |  |  |  |  |
| Sample platform |  |  |  |  |  |  |
| Type | Motor driven multi position sample platform |  |  |  | Motorised camera positioning |  |
| Filter selector |  |  |  |  |  |  |
| Filter positions | 7 - Motor driven |  | 7 - Motor driven |  |  |  |
| Lighting options |  |  |  |  |  |  |
| Epi-white light | Yes |  | Yes |  |  |  |
| HI-LED (red, blue, green) | Optional |  | Optional |  |  |  |
| HI-LED (red, infrared) | Optional |  | Optional |  |  |  |
| HI-LED (red, blue, green, infrared) | Optional |  | Optional |  |  |  |
| Blue converter screen | Optional |  | No |  |  |  |
| Visible light converter | Optional |  | Optional |  |  |  |
| Transilluminator | Optional for 302nm, 365nm, 254nm |  | No UV – green light only |  |  |  |
| Hardware |  |  |  |  |  |  |
| Dimensions (WxHxD cms) | 40x64x52cm |  |  |  | 38x60x68cm |  |
| Operating System | Option for external PC or mounted touch screen PC |  |  |  | Inbuilt PC |  |
| Software |  |  |  |  |  |  |
| Image acquisition | Yes |  | Yes |  |  |  |
| Analysis | Yes – unlimited free copies of GeneTools |  | Yes |  |  |  |
| Warranty |  |  |  |  |  |  |
|  | 3 years |  | 1 year |  |  |  |